| Name: | Basic Electronic Circuits | Semester: | III |
| --- | --- | --- | --- |
| Date of Performance: | 14/9 | Batch No: | B2 |
| Faculty Name: | BPK | Roll No: | 1912052 |
| Faculty Sign & Date: |  | Grade/Marks: | /25 |

Experiment No: 5

Title: Study of BJT Characteristics

Aim and Objective of the Experiment: Study of BJT Characteristics

1. Study CB, CE, and CC Characteristics of NPN 2. Study CB,CE and CC Characteristics of PNP

COs to be achieved:

CO2

Theory:

Transistors are three terminal active devices made from different semiconductor materials that can act as either an insulator or a conductor by the application of a small signal voltage. The transistor’s ability to change between these two states enables it to have two basic functions: “switching” (digital electronics) or “amplification” (analogue electronics). Then bipolar transistors have the ability to operate within three different regions: - Active Region – the transistor operates as an amplifier and Ic = β*Ib - Saturation – the transistor is “Fully-ON” operating as a switch and Ic = I(saturation) - Cut-off – the transistor is “Fully-OFF” operating as a switch and Ic = 0

Circuit Diagram/ Block Diagram:

NPN CB CE CC PNP CB CE CC

Stepwise-Procedure:

- Open a new Schematic. - Draw the Circuit As Shown. - Note down the parameters as per the observation table.

Configuration: CB input characteristics Parameters; VCC = 0V; VCC = 5V; VCCC = 10. NPN Input current (IB) @ VEE = 0 v; 0A; 0A; 0A. PNP Input current (IB) @ VEE = 0 v; 0A; 0A; 0A. NPN Input current (IB) @ VEE = 2 v; 91mA; 92mA; 294.65396mA. PNP Input current (IB) @ VEE = 2 v; 54mA; 67.2mA; 100.95246mA. CB output characteristics Parameters; VEE = 0V; VEE = 1V; VEE = 2; Region of operation. NPN Output current (IC) @ VCC = 0 v; 0A; 0.355A; 1.352A. PNP Output current (IC) @ VCC = 0 v; 0A; 0.284A; 1.492A. NPN Output current (IC) @ VCC = 5 v; 0A; 0.363A; 2.007A. PNP Output current (IC) @ VCC = 5 v; 0A; 0.305A; 1.603A. NPN Output current (IC) @ VCC = 10 v; 0A; 0.371A; 2.046A. PNP Output current (IC) @ VCC = 10 v; 0A; 0.316V; 1.660A. CE Input characteristics Parameters; VCC = 0V; VCC = 5V; VCCC = 10. NPN Input current (IE) @ VEE = 0 v; 0A; 0A; 0A. PNP Input current (IE) @ VEE = 0 v; 0A; 0A; 0A. NPN Input current (IE) @ VEE = 2 v; 0.5A; 2.01A; 2.1A. PNP Input current (IE) @ VEE = 2 v; 0.4A; 1.65A; 1.7A. CE output characteristics Parameters; VBB = 0V; VBB = 1V; VBB = 2; Region of operation. NPN Output current (IC) @ VCC = 0 v; 0A; 0.055A; 0.316A. PNP Output current (IC) @ VCC = 0 v; 0A; 0A; 0A. NPN Output current (IC) @ VCC = 5 v; 0A; 0.364A; 1.986A. PNP Output t current (IC) @ VCC = 5 v; 3.367A; 2.636A; 1.863A. NPN Output current (IC) @ VCC = 10 v; 0V; 0.369A; 2.032A. PNP Output current (IC) @ VBB = 10 v; 7A; 6.306A; 5.603A. CC Input characteristics Parameters; VEE = 0V; VEE = 5V; VEE = 10. NPN Input current (IB) @ VBB = 0 v; 0A; 0A; 0A. PNP Input current (IB) @ VBB = 0 v; 0A; 0A; 0A. NPN Input current (IB) @ VBB = 2 v; 91mA; 100mA; 862.87928mA. PNP Input current (IB) @ VBB = 2 v; 80mA; 88mA; 830.12615mA. CC output characteristics Parameters; VBB = 0V; VBB = 1V; VBB = 2; Region of operation. NPN Output current (IE) @ VEE = 0 v; 0A; 0.079A; 0.536A. PNP Output current (IE) @ VEE= 0 v; 0A; 0A; 0A. NPN Output current (IE) @ VEE = 5 v; 0A; 0.367A; 2.101A. PNP Output current (IE) @ VEE = 5 v; 0.830A; 1.022A; 1.176A. NPN Output current (IE) @ VEE = 10 v; 0A; 0.378A; 2.133A. PNP Output current (IE) @ VEE = 10 v; 1.745A; 1.855A; 1.964A.

Calculation:

1. Rin 2. Ro

Waveform

All input and output characteristics(12) NPN CB CE

CC PNP CB CE CC

Post Lab Subjective/Objective type Questions: (hand written)

Characteristic

Input Dynamic Resistance

Output Dynamic Resistance

Current Gain

Voltage gain

Power gain

Leakage current

Common base (CB)

Very Low(less than 100 ohm)

Very High

Less than 1

Greater than CC but less than CE

Medium

Very small

Common emitter (CE)

Low(less than 1K)

High

High

Highest

Highest

Very large

Common collector (CC)

Very High(750K)

Low

Very High

Lowest(less than 1)

Medium

Very large

In phase 2. Comparison of NPN and PNP mm The NPN and PNP both are the bipolar junction transistor. It is the current controlling device and mainly used for switching and amplifying the signal. Mostly, the NPN transistor is used in the circuit because in NPN transistors the conduction current is mainly by electrons while in the PNP transistor the conduction current is because of the holes. As the electrons are more mobile the NPN has high conduction. The letter PNP and NPN show the voltage required by the emitter, collector and base of the junction transistor. The NPN and PNP transistor, both are made up of different material due to which the current developed in them also differs. Sometimes when the voltage applied across the emitter the electrons cross the base junction and reach the collector region. This happens because the base of the NPN and PNP transistor is very thin and lightly doped.

Conclusion: (to be written in own words)

In this experiment we studied the CB, CE and CC characteristics of NPN and PNP transistors and plotted waveforms for the same.

Signature of faculty in-charge with Date: