| Course Name: | Linear Integrated Circuits and Design | Semester: | V |
| --- | --- | --- | --- |
| Date of Performance: | 18/08/2020 | Batch No: | B1 |
| Faculty Name: | Prof. Milind Marathe | Roll No: | 1912052 |
| Faculty Sign & Date: |  | Grade/Marks: | /25 |

Experiment No: 1

Title: To implement Subtractor using opamp

Aim and Objective of the Experiment:

To implement Linear equations using op-amp IC 741. - To design and implement Linear equations using op-amp IC 741.

COs to be achieved:

CO2: Design circuits using op-amps as linear applications.

Theory:

The uA741 is used to solve different linear equation. The linear equation implemented as: From figure, Vout=Va-Vb

Circuit Diagram:

Stepwise-Procedure:

1. Connections are given as per the circuit diagram. 2. + Vcc and - VEE supply is given to the power supply terminal of the Op-Amp IC. 3. Apply input voltages as per requirement. 4. Observe the output voltage obtained on the CRO.

Observation Table:

Sr. No.; Vb; Va; Theoretically value Vout; Simulated values Vout. 1.; 3; 1; 2; 1.9999946V. 2.; 1; 3; -2; -1.9999946V. 3.; 5; 4; 1; 0.9999732V. 4.; 5; 5; 0; 35.90nV. 5.; 7; 4; 3; 3.9999919V. 6.; 13; 1; 12; 11.035419V. 7.; 20 (p-p) Sine; 4; -; Screenshots attached. 8.; 10(Sq. Wave); 5; -; Screenshots attached. 9.; 10 Sq; 10 p-p(Sine Wave); -; Screenshots attached. 10.; -6; 6; -12; -11.00993V. With Different R values Sr. No.; Vb; Va; Theoretically value Vout; Simulated values Vout. 1.; 6; 3; 1.667V; 1.666V. 2.; 3; 3; 0.333V; 0.333V. 3.; -4; 4; 3.109776V; 3.111V. 4.; 9; 6; 1.999V; 1.999V. For Different R

Calculation:

Design:- Vo=R4R2*V1-R1R3*V2 …..(1) - To design Subtractor, R1=R2=R3=R4 So that equation 1 becomes Vo=(V1-V2) Assume, R1=R2=R3=R4=10KΩ

Waveforms:

Circuit All Rs=5ohms

For Different values of R

Calculationsss

Post Lab Questions:

- List of all linear applications of op-amp. Ans: - Sign changer, scale changer, inverting, and non-inverting amplifier. - Integrator, differentiator, and its application in analog computer. - Current to voltage (C–V or C/V ) and voltage to frequency (V–F or V/F ) circuits. - Voltage to frequency (V–F or V/F ) and frequency to voltage (F–V or F/ V ) conversion circuits. - Different circuits using op amps are analysed with input and output signal waveforms. - Design averaging circuit for four DC inputs. V(vout_vedant): -11 voltage - Design Adder-Subtractor circuit using opamp. Vout=V3+V4-(V1+V2) V(vout_vedant): -8.99996 voltage

Conclusion:

Linear equations using IC 741 is successfully implemented. Thus we have implemented linear equations such as subtractor, averaging circuit and adder-subtractor using Opamp and verified it with the given input signals.

Signature of faculty in-charge with Date: